Practical 1

Setup of basic lan

To set up a basic Local Area Network (LAN) for Practical 1, you can follow these steps which cover both physical implementation and software simulation (such as Cisco Packet Tracer).

Objective

To establish a basic wired LAN using multiple computers and a switch, assign IP addresses, and verify connectivity.

Requirements

- Hardware: At least 2–4 PCs with Network Interface Cards (NICs), 1 Layer 2 Switch (e.g.,

Cisco 2960

), and Copper Straight-Through Ethernet cables (Cat 5e or Cat 6).

- Software (for simulation): Cisco Packet Tracer.

Procedure (Simulation & Physical)

- Design the Topology:

  - Place a central Switch in the workspace.

  - Place 3 or 4 PCs around the switch.

- Make Physical Connections:

  - Connect each PC's Ethernet port to an available port on the switch using a Straight-Through cable.

  - Wait for the link lights to turn green (indicating a physical link is active).

- Assign IP Addresses:

  - Manually configure a static Class C IP address for each device (e.g., using the 192.168.10.x range):

    - PC0: IP 192.168.10.1, Subnet Mask 255.255.255.0

    - PC1: IP 192.168.10.2, Subnet Mask 255.255.255.0

    - PC2: IP 192.168.10.3, Subnet Mask 255.255.255.0

- Verify Configuration:

  - On any PC, open the Command Prompt (CMD).

  - Type ipconfig and press Enter to verify your assigned IP address.

- Test Connectivity (Ping Utility):

  - From PC0, attempt to reach PC1 by typing: ping 192.168.10.2.

  - A successful connection is confirmed if you receive "Reply from..." messages.

Troubleshooting Tips

- Physical Layer: Ensure cables are firmly plugged in and the switch is powered on.

- Network Layer: Ensure all devices are on the same subnet (the first three octets of the IP address must be identical).

- Firewall: If using physical Windows PCs, you may need to disable the Windows Firewall to allow "ping" requests to pass through.